TYPICAL SPECIFICATION

HYDRAULIC CENTURY®

CHEMICAL TOTE BIN SCALE

SPECIFICATION SECTION 11245 - Chemical Feeders

SPECIFICATION SECTION 11240  -  Chemical Metering Pumps

Platform scales shall be of the hydraulic cell type.  Scale shall be sized to accept up to a     _______ inch (mm.) diameter tank.     The low profile platform shall include an adjustable backstop to accommodate easy loading and unloading of chemical tote bins.    Platform scale coating system shall be a minimum dry thickness of 80 mils and be resistant to moisture, chemicals, abrasion, impact and UV light.

Load cell shall be of the temperature-stable, rolling diaphragm type.   Load plate shall be able to tilt to 4 degrees without affecting accuracy to allow easy installation.  Flexible hose shall lead from the load cell to allow easy remote installation of the dial and shall be ________ft. (meter) in length, 12ft (4M) standard.   Load cell system shall require no electric power and shall be immune to all RFI/EFI, power failures and lightening strikes.

Dial diameter shall be ______ inches and read zero to _______ lbs.  (kgs) with provision for tare adjustment.  Dial shall be temperature stable with damper installed to prevent shock damage.

Scale shall carry a minimum of a Five (5) Year Factory Warranty.    “Limited” Warranties shall be considered unacceptable.

Full scale accuracy shall be better than ½ of 1%.   Scale shall be CHEM-SCALETM TOTE BIN SCALE with TUF-COATTM coating system, hydraulic CENTURY® dial indicator, Model  ____ - 12D_____-TB,   as manufactured by FORCE FLOW, 2430 Stanwell Drive, Concord, CA 94520 USA (www.forceflow.com), or equal.

OPTION 1:	Scale shall have a SATELLITE® loop powered 4-20mA output proportional to gross weight.

OPTION 2:	Scale shall have a 5amp low level contact with manual reset button.

OPTION 3:	Scale shall have a Fixed Alarm 10amp Hi/Low Pressure Switch.

Model Numbers:

PLATFORM & INDICATOR:

Capacity lbs	50" x 50" 	60" x 60"

3,000 lb.	50-12D30TB	60-12D30TB

4,000 lb.	50-12D40TB 	60-12D40TB

5,000 lb.	50-12D50TB	60-12D50TB

6,000 lb.	50-12D60TB	60-12D60TB

Consult factory for metric model numbers

File:   _WEB\SPECS\SPCTBHY   02/03/03

FORCE FLOW  1-800-893-6723

FAX: 925-686-6713

CHEMTB/HYD